# Ryan C. Balaguer (Slides 1-5) - ~7 Minutes

## Slide 1 – Title (1 minute)

Good morning, panel members, judges, managers, and colleagues.

We are honored to present our Quality Symposium project entitled "Preventing the Use of Expired Epoxy Through Smart Automation in the MCA Process on Essemtec Machines."

I am Ryan Balaguer, together with Jake Vincent Basalo and Marc Lorenz Coruña from the Assembly Maintenance Team.

Today, we will discuss a quality risk associated with epoxy usage, our investigation into its root causes, the automation solution we developed, and the benefits it brings to our manufacturing process.

## Slide 2 – Authors (30 seconds)

This project is the result of a collaborative effort among our team members.

Each member contributed to the problem analysis, design, programming, installation, testing, and validation of the system.

Through teamwork and technical collaboration, we successfully developed a practical and cost-effective automated solution.

## Slide 3 – Background of the Study (2 minutes)

In the MCA process, epoxy is used to attach magnets onto semiconductor packages.

As we know, epoxy has a limited working life. Once it exceeds its allowable usage period, its bonding effectiveness may decrease.

Currently, operators manually monitor epoxy expiration time.

While operators follow established procedures, manual monitoring is still vulnerable to human error. During periods of high workload, expiry checking can potentially be overlooked.

The Essemtec machine does not have a built-in smart monitoring feature that can warn operators or stop production when epoxy has expired.

Because of this, we saw an opportunity to strengthen our process through automation and error-proofing.

Our goal was not to replace the operator, but rather to provide an additional layer of protection that ensures expired epoxy can never be used unintentionally.

## Slide 4 – Statement of the Problem (1.5 minutes)

The primary problem identified was the dependence on manual verification of epoxy working life.

In the existing process:

- No automatic timer exists.

- No warning alarm is available.

- No machine interlock prevents continued operation.

- Monitoring depends entirely on human intervention.

This creates a risk where expired epoxy may accidentally remain in use.

If this occurs, it may affect bond integrity, product quality, and process reliability.

Therefore, our challenge was to develop a solution that automatically monitors epoxy life while eliminating the possibility of oversight.

## Slide 5 – Objectives (2 minutes)

To address the problem, we established the following objectives:

First, prevent the use of expired epoxy.

Second, provide advance notifications before expiration occurs.

Third, reduce dependence on manual monitoring.

Fourth, improve process reliability and product quality.

Fifth, minimize defects associated with epoxy degradation.

And finally, improve operational efficiency by allowing operators to focus on production activities while the system performs monitoring automatically.

By achieving these objectives, we aimed to strengthen both quality control and process sustainability.

At this point, I will turn over the presentation to Jake, who will discuss our root cause investigation and solution development.

# Jake Vincent Basalo (Slides 6-10) -

## Slide 6 – Fishbone Analysis (2 minutes)

Thank you, Ryan.

To understand the source of the problem, we conducted a Fishbone Analysis.

We categorized potential causes into four major areas:

### Man

- Failure to check expiration time

- Human error

- Forgetfulness

### Machine

- No timer

- No alarm

- No monitoring function

### Method

- Reliance on manual recording

- Lack of automated controls

### Material

- Limited epoxy working life

- Sensitivity to time exposure

After evaluating all contributing factors, we determined that the most significant root cause was the absence of automated monitoring and machine controls.

Therefore, our improvement effort focused on addressing this technical gap.

## Slide 7 – Why-Why Analysis (1.5 minutes)

To further validate our findings, we performed Why-Why Analysis.

We asked why expired epoxy could still be used.

The answer was because operators manually monitor expiration.

Why is manual monitoring needed?

Because no automated monitoring system exists.

Why is there no automated monitoring?

Because the machine was not originally designed with this capability.

As we continued the analysis, it became clear that implementing automation would directly eliminate the root cause.

The analysis confirmed that process control, rather than operator behavior, should be the focus of improvement.

## Slide 8 – Failure Mechanism (1 minute)

This slide illustrates the effects of epoxy aging.

Fresh epoxy provides the required bonding performance necessary for reliable magnet attachment.

As epoxy exceeds its usable working life, material characteristics may change, reducing bonding effectiveness.

This can increase the risk of poor adhesion and quality-related issues.

This demonstrates why strict work-life control is critical in the MCA process.

## Slide 9 – Proposed Solution (1.5 minutes)

To eliminate the root cause, we developed the Smart Automated Epoxy Timer System.

Using the HMI screen, operators enter the allowable working life of the epoxy.

The system then performs continuous countdown monitoring.

Before the epoxy expires, advance warnings are generated.

Upon expiration, the machine automatically enters an inhibited state, preventing further operation.

This ensures that expired epoxy cannot be used in production.

Instead of relying solely on human monitoring, the process is now protected through automation.

## Slide 10 – Features of the System (1.5 minutes)

Our Smart Epoxy Timer System includes several key features.

These include:

- HMI touchscreen interface

- Countdown timer display

- Visual alerts

- Audible alarms

- User access control

- Machine interlock protection

- Power interruption recovery

- Self-diagnostic capability

Together, these features provide a reliable and user-friendly system for monitoring epoxy work life.

I will now hand over the presentation to Marc to discuss the features and results of the implemented solution.

# Marc Lorenz Coruña (Slides 11-16) -

## Slide 11 – Results and Benefits (2 minutes)

Thank you, Jake.

After implementation, we observed several significant improvements.

The system now continuously monitors epoxy life without requiring manual calculations.

Operators receive clear visual and audible notifications.

Most importantly, machine operation can be automatically inhibited when epoxy reaches expiration.

From a quality perspective, the solution reduces the risk of expired epoxy usage.

From an operational perspective, it improves consistency and reduces dependence on human intervention.

From a financial perspective, the results are also significant.

A commercially available solution was estimated to cost approximately ₱1.9 million.

Through internal development, our team completed the project for approximately ₱75,000.

This generated an estimated cost avoidance of approximately ₱1.825 million.

## Slide 12 – Conclusion (1 minute)

In conclusion, the project successfully addressed the identified quality risk.

Through smart automation, alarm notification, and machine interlock functionality, we eliminated the possibility of unintentionally using expired epoxy.

The solution improved quality, reliability, efficiency, and overall process control.

Furthermore, it demonstrated how internally developed automation can create substantial value at minimal cost.

## Slide 13 – Standardization (45 seconds)

The system has been successfully installed and validated on MCA-027.

To ensure sustainability, we are preparing supporting documentation, operating guidelines, and standard procedures.

These measures will support long-term implementation and knowledge transfer.

## Slide 14 – Recommendation (45 seconds)

Moving forward, we recommend continued monitoring and validation of the system.

We also recommend expanding implementation to other applicable Essemtec machines.

Additionally, the same concept can potentially be adapted for other manufacturing processes requiring time-based material control.

## Slide 15 – Acknowledgment (30 seconds)

We would like to sincerely thank our managers, engineers, technicians, trainers, and all those who supported this project.

Their guidance, expertise, and encouragement were vital to the successful completion of this initiative.

## Slide 16 – Closing (30 seconds)

This concludes our presentation.

Thank you for your time and attention.

We welcome any questions, comments, or recommendations from the panel.

## Strong Closing Line Before Q&A

Marc should end with:

"This project demonstrates that a simple, low-cost automation solution can effectively eliminate human error, improve product quality, and generate significant cost savings. Thank you, and we are now ready for your questions."

That closing statement usually leaves a strong impression on Quality Symposium judges and management panels.